## Implementation of a Cache Controller using a HDL

### 1. Project Overview

This project requires students to design, implement, and test a cache controller for a simple computer system using a hardware description language (HDL). The project will be handled by the same teams that implemented the ALU. The cache controller will be built around a finite state machine (FSM) to manage the operations and state transitions effectively. This exercise aims to deepen students' understanding of cache memory management, FSM-based design, and HDL programming, which are critical in computer engineering and architecture.

### 2. Learning Objectives

Students will achieve the following by completing this project:

- Develop a comprehensive understanding of cache memory mechanisms and their impact on system performance.

- Gain practical experience in designing a hardware module based on FSM principles.

- Implement and simulate a set-associative cache controller using an HDL.

- Evaluate the performance and efficiency of the cache system through rigorous testing.

### 3. Project Description

#### 3.1 Cache Controller Specifications

The cache controller to be designed must adhere to the following specifications:

- Cache Type: 4-way set associative

- Cache Size: 32 KB

- Block Size: 64 bytes

- Number of Sets: 128 sets

- Associativity Level: 4-way

- Replacement Policy: Least Recently Used (LRU)

- Write Policy: Write back with write allocate

#### 3.2 FSM-Based Design

The cache controller should be implemented as a finite state machine with clearly defined states including, but not limited to:

- IDLE: Waiting for a new request.

- READ HIT: Accessing data for a read request found in the cache.

- READ MISS: Handling read requests when data is not in the cache.

- WRITE HIT: Managing write operations when data is found in the cache.

- WRITE MISS: Processing write operations when data is not found in the cache.

- EVICT: Evicting data from the cache according to the LRU policy.

Each state must handle the transitions based on the cache operation required and update the cache accordingly.

#### 3.3 Hardware Description Language

Students may select one of the following HDLs:

- VHDL

- Verilog

- SystemVerilog

#### 3.4 Simulation and Testing Tools

Appropriate tools for simulation and validation must be used, such as:

- ModelSim for wave simulation

### 4. Deliverables

#### 4.1 Detailed Design Specification

This document must include:

- Detailed FSM diagram showing all states and transitions

- Functional block diagram of the cache controller

- Description and justification of the chosen HDL

#### 4.2 Implementation

The complete HDL code for the cache controller, including:

- Modular HDL code for each component of the cache controller

- In-line comments explaining critical sections of the code

#### 4.3 Test Bench and Simulation Results

A detailed test plan and the corresponding simulation results must be presented, illustrating:

- Various test cases covering all possible states and transitions

- Waveforms and state transitions observed during simulations

- Performance metrics such as hit rate and access time

#### 4.4 Final Report

#### The project culminates in a comprehensive report that covers:

- Overview of the design and implementation process

- Technical challenges encountered and solutions implemented

- Analysis of the performance data collected during simulations

### 5. Evaluation Criteria

Projects will be evaluated based on:

- Technical complexity and correctness of the FSM design

- Functionality and robustness of the HDL implementation

- Completeness and depth of the testing process

- Quality of documentation

6. Timeline

The project spans 4 weeks, and is to be handed in by email at bozdogalex@gmail.com, the due date being 31.05.2024, 24:00.

Any questions will be discussed during the laboratory.